## 0) Mental model: SAC vs ObsPy

- SAC: you load one or more traces into memory, then run commands that modify the current in-memory data and header. Great for quick interactive work and repeatable macros.

- ObsPy: you load data into Stream/Trace objects, then call Python methods/functions. Great for pipelines, automation, metadata handling, and integration with the rest of Python.

SAC is what seismologists used a generation ago, so lots of mid-career and older faculty use it. It had to be used in conjunction with shell scripting, e.g. bash or perl. ObsPy is what seismologists use today, and you can build entire workflows in Python.

## 1) Read data

### SAC

r XX.STA..BHZ.SAC

lh npts delta b e kstnm kcmpnm

### ObsPy

from obspy import read

st = read("XX.STA..BHZ.SAC")   # also reads MiniSEED, etc.

tr = st[0]

print(tr.stats.npts, tr.stats.delta, tr.stats.starttime, tr.stats.channel)

## 2) Quick plot

### SAC

r *.SAC

p1

### ObsPy

from obspy import read

st = read("*.SAC")

st.plot()

## 3) Trim a time window (cut)

### SAC

r XX.STA..BHZ.SAC

cut 2026 041 12 00 00 0  2026 041 12 05 00 0

r

cut off

p1

### ObsPy

from obspy import read, UTCDateTime

st = read("XX.STA..BHZ.SAC")

t0 = UTCDateTime("2026-02-10T12:00:00")

st.trim(t0, t0 + 5*60)

st.plot()

## 4) Demean / detrend / taper

### SAC

r XX.STA..BHZ.SAC

rmean

rtr

taper

p1

### ObsPy

st = read("XX.STA..BHZ.SAC")

st.detrend("demean")

st.detrend("linear")

st.taper(max_percentage=0.05)

## 5) Bandpass filter

### SAC

r XX.STA..BHZ.SAC

bp c 1 10 n 4 p 2

p1

### ObsPy

st = read("XX.STA..BHZ.SAC")

st.filter("bandpass", freqmin=1.0, freqmax=10.0, corners=4, zerophase=True)

st.plot()

Notes:

- SAC’s p 2 corresponds to zero-phase (forward/backward) filtering → zerophase=True in ObsPy.

## 6) Resample / decimate

### SAC

r XX.STA..BHZ.SAC

decimate 2

lh delta

### ObsPy

st = read("XX.STA..BHZ.SAC")

st.decimate(factor=2, no_filter=False)  # keep anti-aliasing filter on

print(st[0].stats.delta)

## 7) Frequency spectrum (FFT magnitude)

### SAC

r XX.STA..BHZ.SAC

fft

p1

### ObsPy

import numpy as np

from obspy import read

tr = read("XX.STA..BHZ.SAC")[0]

data = tr.data.astype(float)

n = len(data)

dt = tr.stats.delta

spec = np.fft.rfft(data)

freq = np.fft.rfftfreq(n, d=dt)

amp = np.abs(spec)

# quick plot

import matplotlib.pyplot as plt

plt.figure()

plt.plot(freq, amp)

plt.xlabel("Hz")

plt.ylabel("|FFT|")

plt.show()

## 8) Pick arrivals (manual workflow)

### SAC (classic picking)

r XX.STA..BHZ.SAC

p1

ppk

wh

### ObsPy (common approach)

ObsPy doesn’t have SAC’s exact interactive picker built-in, but typical workflows are:

- Use a viewer/picker (e.g., ObsPy’s plotting + external tools), then store picks in a Catalog/Event (QuakeML), or

- Use an auto-picker (STA/LTA, AIC, etc.) then review.

Example: simple STA/LTA trigger (not a “picker”, but often used as a start):

from obspy import read

from obspy.signal.trigger import classic_sta_lta, trigger_onset

tr = read("XX.STA..BHZ.SAC")[0]

cft = classic_sta_lta(tr.data, int(1/tr.stats.delta), int(10/tr.stats.delta))

on_off = trigger_onset(cft, 3.5, 1.0)

print(on_off[:10])  # sample indices for trigger on/off

## 9) Instrument response removal

### SAC (two common cases)

A) Using SAC PoleZero

r XX.STA..BHZ.SAC

rmean; rtr; taper

trans from polezero subtype STATION.PZ to vel freqlimits 0.01 0.02 30 40

B) Using RESP

r XX.STA..BHZ.SAC

trans from evalresp fname RESP.NET.STA.LOC.CHA to vel freqlimits 0.01 0.02 30 40

### ObsPy (recommended: Inventory + remove_response)

from obspy import read, read_inventory

st = read("XX.STA..BHZ.mseed")  # or SAC

inv = read_inventory("station.xml")  # or RESP via read_inventory if supported in your setup

st.detrend("demean")

st.taper(0.05)

st.remove_response(inventory=inv, output="VEL",

pre_filt=(0.01, 0.02, 30.0, 40.0),

water_level=60)

## 10) Rotate components (NE→RT, ZNE handling)

### SAC (requires correct headers: CMPAZ/CMPINC etc.)

r XX.STA..BH?.SAC

rotate to gcp

# or: rotate NE to RT backaz <value>

### ObsPy

from obspy import read

from obspy.signal.rotate import rotate_ne_rt

st = read("XX.STA..BH?.SAC")

# assume you’ve got N and E traces

trN = st.select(channel="BHN")[0]

trE = st.select(channel="BHE")[0]

baz = 123.0  # back-azimuth in degrees (station -> event)

r, t = rotate_ne_rt(trN.data, trE.data, baz)

## 11) Write output

### SAC

r XX.STA..BHZ.SAC

bp c 1 10 n 4 p 2

w XX.STA..BHZ.bp_1_10.SAC

### ObsPy

st = read("XX.STA..BHZ.SAC")

st.filter("bandpass", freqmin=1, freqmax=10, corners=4, zerophase=True)

st.write("XX.STA..BHZ.bp_1_10.SAC", format="SAC")

## 12) Batch processing

### SAC macro (skeleton)

Create process.m:

echo on

wild echo off

r *.SAC

rmean

rtr

taper

bp c 1 10 n 4 p 2

w over

q

Run:

sac process.m

### ObsPy script (skeleton)

from glob import glob

from obspy import read

for fn in glob("*.SAC"):

st = read(fn)

st.detrend("demean")

st.detrend("linear")

st.taper(0.05)

st.filter("bandpass", freqmin=1, freqmax=10, corners=4, zerophase=True)

st.write(fn, format="SAC")  # overwrite

## Quick “when to use which”

- Use SAC when you want fast, interactive, “poke-and-see” waveform work, quick manual edits, classic workflows, or you’re collaborating with people entrenched in SAC macros.

- Use ObsPy when you want reproducible pipelines, many-station processing, rich metadata (StationXML/QuakeML), integration with pandas/NumPy, ML feature extraction, and modern packaging/testing.