## Homework: SEISAN → MiniSEED + SAC (metadata + preprocessing workflow)

### Goal

You will take a single SEISAN waveform file:

2001-02-02-0303-55S.MVO___019

(this is in the week5 folder)

and convert it into processed MiniSEED and SAC outputs, using ObsPy. Along the way you will practice:

- parsing information from filenames

- reading SEISAN waveforms

- editing Trace metadata

- filtering + tapering

- unit scaling using a sensitivity

- resampling

- writing MiniSEED and SAC

## Requirements

### 1) Parse the filename

Given the input filename, extract:

- Start time from the prefix formatted as YYYY-MM-DD-hhmm-ss

Example: 2001-02-02-0303-55 → 2001-02-02T03:03:55

- The format code: the single letter after the time is S meaning SEISAN.

- The number of channels: the last 3 digits (019) → integer number of channels.

Print a one-line summary like:

Parsed starttime=…, format=…, nchans=…

### 2) Read the SEISAN file

Read the file into an ObsPy Stream.

- Verify that the number of traces in the stream matches the parsed channel count.

- If it does not match, print a warning and continue.

### 3) Edit metadata: network code

For every Trace in the Stream, change network code from:

- MV → MT

(Leave station/location/channel unchanged.)

### 4) Preprocess each trace

Apply these steps in this order:

- Taper: 5% taper

- High-pass filter: 0.5 Hz, two-way (zero-phase)

- Scale to physical units using an overall sensitivity:

  - 8,000,000 counts per (m/s)

  - Convert counts → m/s by:

v = \frac{\text{counts}}{8{,}000{,}000}

In code: tr.data = tr.data / 8_000_000.0

### 5) Resample to 100 Hz

Change each trace sampling rate to 100 samples/sec.

### 6) Force the start time

Set the start time for every trace to:

UTCDateTime(‘2001-02-02T03:00:00’)

### 7) Write outputs

Write two outputs:

A) MiniSEED

- Output filename should match the input naming style, but replace the S with M. But remember that the start time has changed.

- Write stream as MiniSEED.

- Re-read the MiniSEED file and plot it.

B) SAC

- Write SAC files (one per trace).

- Put them in a folder like SAC/.

- Filenames should include NET.STA.LOC.CHA and optionally start time.

- Re-read the SAC files and plot them.

## Deliverables

Submit (I will make this a Canvas assignment):

- Your Python script or notebook (hw_seisan_convert.py or .ipynb)

- Screenshots of the stream plots of the MiniSEED and SAC files you read back in.